# CE3005 Lecture Notes

# Network Layers and Physical Resilience

## Layered Network Architecture

- Network is organized as a stack of layers

  - Each layer offers services to the layer above, and processes information from the layer below, using an interface

  - Allows ease of design – other layers aren’t affected by changes to one layer

- Protocol: set of rules that govern communication between two peering parties (computers)

  - Format of messages

  - Order of messages

  - Actions that should be taken upon transmitting/receiving of messages

- Network architecture: set of layers and protocols that enable developers to build systems

## OSI (Open Systems Interconnection) 7-Layer Model

- Message starts off at application layer, moves down to physical layer

- Each layer adds a header to the message it receives – layer-specific information

- At the receiving end, headers are stripped as the message travels up the OSI layers

### Peer Processes

- Layer n in one machine interacts with layer n in another to provide services to layer n+1

- Peer processes are the entities that contain the corresponding layers on different machines

  - They utilize the layer-n protocol

  - They communicate through exchanging Protocol Data Units (PDUs)

### Services

- Layer n+1 transfers information by invoking services on layer n, which are available from Service Access Points (SAPs)

  - Each layer passes data to the layer below, until the physical layer where transfer occurs

  - Data is passed between layers as Service Data Units (SDUs), which encapsulate PDUs

## TCP/IP Model

- Application: supporting network applications (FTP, SMTP, HTTP)

- Transport: host-to-host data transfer (TCP, UDP)

- Network: routing of data from source to destination (IP, routing protocols)

- Link: data transfer between neighbouring network elements (PPP, Ethernet)

- Physical: bits on the wire

- Each layer takes data from the layer above and adds a header before passing data to layer below

## OSI vs. TCP/IP

# Physical Layer: Network Resilience

## Network Reliability

- The probability that a network performs satisfactorily over a period of time

MTBF=MTTF-MTTR

  - MTBF = mean time between failures

  - MTTF = mean time to failure

  - MTTR = mean time to repair

## Link Failure Probability & Availability

- Link failure probability bi: percentage of time where link does not work

- Link availability ri: percentage of time that link works

- ri=1-bi

## Network Resilience

- Measures how tolerant a network is to faults, expressed based on probability that the network remains connected

### Single Link

If the probability of a break is 0.05, then:

- bSG-AU=0.05

- rSG-AU=1-bi=0.95

### Series Links

For links in series, calculate the probability that all links are working

If the probability of a break is 0.05, what is the probability that SG cannot communicate with AU?

- P(SG cannot communicate with AU) = 1 – P(SG can communicate with AU)

  - bSG-HW=bHW-AU=0.05

  - rSG-HW=rHW-AU=0.95

  - rSG-AU=rSG-HW*rHW-AU=0.95*0.95=0.9025

  - bSG-AU=1-0.9025=0.0975

### Parallel Links

For links in parallel, calculate the probability that all links are broken

If the probability of a break is 0.05, what is the probability that SG cannot communicate with AU?

- P(SG cannot communicate with AU) = P(both links break)

  - bsingle link=0.05

  - bSG-AU=bsingle link*bsingle link=0.05*0.05=0.0025

### Hybrid Paths

For hybrid paths, decompose into paths and multiply the probabilities

If the probability of a break is 0.05, what is the probability that SG cannot communicate with AU?

- P(SG cannot communicate with AU) = P(both paths break)

  - bSG-HW-AU=0.0975

  - bSG-AU=0.05

  - bSG-AU=bSG-HW-AU*bSG-AU=0.0975*0.05=0.004875

# Data Link Layer (DLL)

## Data Link Layer Services

- Framing: encapsulate (network-layer) datagram in a link-layer frame before transmission over link

- Link Access: MAC protocol specifying rules by which frames are transmitted onto link

- Flow Control: control data flow to ensure all data is received properly

- Reliable Delivery: move datagram over link without errors

## Framing

### Byte/Character-Oriented

- Information is framed into a fixed 8-bit unit

  - 1 byte = 1 ASCII code

  - Some bytes used for signalling/protocol control

- Useful when technology was primitive

### Bit-Oriented

- Flag bits are used to frame the data bits

- Header/trailer used to describe frame

  - Some frames used for control

- Used by modern protocols (HDLC, PPP, Ethernet, etc.)

## Link Access

- Determines who gets to transmit when on a link

- Topology: physical arrangement of stations

  - Point-to-Point: hosts are directly connected in pairs – multiple direct transmissions

  - Broadcast: all hosts connected to single channel – everyone receives all messages

- Duplexity

  - Half Duplex: only one party can transmit at a time

  - Full Duplex: allows simultaneous transmission and reception between two parties

# Flow Control

- Ensures that the transmitting party doesn’t overwhelm the receiving party with data

## Stop-and-Wait

- A puts data in a frame, sends frame to B

- A waits for ACK from B

- B receives data, sends ACK

- A receives ACK, repeat

### Performance

Throughput (U)=time link carries useful informationtotal time=TframeTcycle

- Tcycle=Tframe+Tprop+Tproc+Tack+Tprop+Tproc

- If we assume that Tack and Tproc are negligible, then:
Tcycle=Tframe+2Tprop

U=TframeTcycle=TframeTframe+2Tprop

U=11+2a,     a=TpropTframe

  - a: Normalized Propagation Delay (unitless)

Example:

- Tframe=frame lengthlink transmission rate =2400 bits2.4×106 bps=1000 μs

- Tprop=distancevelocity=50000 m2×108 m/s=250 μs

- U=11+2TpropTframe=11+22501000=23

### Disadvantages

- If frame or ACK is lost (error occurs), long wait time expected

  - Can use timeout, but this is not prime

- If propagation time is long, sender must wait long time to transmit next data

  - Can use buffer – sliding window

## Sliding Window

- Allows multiple frames to be in transit at the same time

- Sender and receiver each have buffer length N

  - Sender can send maximum N frames without receiving ACKs

  - Each frame is numbered - ACK indicates number of next expected frame

- Sender:

  - Moves lower bound of buffer when frame sent

  - Moves upper bound of buffer when ACK received

- Receiver:

  - Moves lower bound of buffer when frame received

  - Moves upper bound of buffer when ACK sent

- If frame number bounded by k bits, then frames are numbered modulo 2k (0 to 2k-1)

  - Window size N must be chosen such that N≤2k

- Other features:

  - Receiver can acknowledge frame and stop further transmission by sending RNR (Receiver Not Ready) frame, and sending a normal ACK when transmission can continue

  - ACK can be piggy-backed on data frames in the reverse direction

### Performance

- Sliding window performance depends on:

  - Window size N

  - a=TpropTframe

- Two cases:

  - N≥2a+1: continuous transmission is possible, so U=1

  - N<2a+1: the window is filled at t=N, and transmission cannot resume until t=2a+1, so U=N1+2a